# How to Write a Literate Programming Protocol

### Simple Rules

## Introduction

This document defines a minimal standard for writing literate protocols in this repository.

Scope: Covers only:

- Workflows

- Unit operations that compose those workflows

## ID Conventions

### Workflow ID Convention

Workflow identifiers follow this format:

W-O-TT-XX

- W – Workflow

- O – Opentrons (platform)

- TT – Assay / test category

- XX – Sequential number within the category

#### Assay Category Codes

- PS – Phosphate solubilisation

- NF – Nitrogen fixation

(Extend as needed)

#### Examples

- W-O-PS-01

- W-O-NF-02

### Unit Operation ID Convention

Unit operations follow:

U-<type>-XX

- U – Unit operation

- type – Category:

  - O – Opentrons robot operation (liquid handling)

  - C – Clariostar operation (e.g., plate reader)

- XX – Sequential number

#### Examples

- U-O-03 (Opentrons operation)

- U-C-02 (Clariostar operation)

## 1. One Notebook = One Workflow

- Each notebook must implement exactly one workflow ID
(e.g. W-O-PS-02)

- Recommended filename format:

W-O-PS-02_phosphate_liquid_assay.ipynb

## 2. Start with a Controlled Header (SOP Block)

The first markdown cell must include:

- Doc No (e.g. SOP W-O-PS-01)

- Version

- Effective date

- Owner

- Approved by

- Workflow ID

- Workflow name

- Unit operations used (in order)

- Output (1 sentence)

This transforms the notebook into a controlled protocol document.

## 3. Unit Operations = Table of Contents

Include early:

### Steps (unit operations)

- U-... Unit operation name

- U-... Unit operation name

- U-... Unit operation name

This should allow understanding the protocol in ~10 seconds.

## 4. Parameters Live in One Place

- Use exactly one "Parameters" section

- Use clear variable names with units, e.g.:
MEDIA_VOL_UL

- Avoid magic numbers elsewhere

## 5. Standard Format for Each Unit Operation

Each section must follow:

### U-XX-YY Unit Operation Name

- Goal: (1 line)

- Inputs: (plates, reagents, files)

- Outputs: (what is produced / where saved)

- Acceptance criteria: (conditions to proceed)

- Record: (what to log)

- Deviation handling: (what to do if failure occurs)

- Notes: (tips, contamination, timing, etc.)

Keep it concise and operational.

## 6. Separate Story vs Execution

For each unit operation:

- 1–3 markdown bullets → describe intent

- Followed by → code cell(s) implementing it

## 7. Always End with Artifacts

At the end of the notebook, clearly show:

- Output file locations (e.g. generated scripts)

- Short run summary (key parameters used)

## 8. Validation Gates (Mandatory Before Release)

Each workflow must define:

- Input validation

  - Required columns

  - Value ranges

  - Duplicate checks

- Output validation

  - Row counts

  - Unique wells

  - Correct mappings

- Script validation
python -m py_compile

- Simulation validation
opentrons_simulate

- Clear pass/fail acceptance criteria

## 9. Controlled Change Log

At the end, include a table with:

- Date

- Version

- Author

- Summary of changes

This ensures traceability and auditability.

## Minimal Template (Copy/Paste)

### SOP Header

- Doc No:

- Version:

- Effective date:

- Owner:

- Approved by:

- Workflow ID:

- Workflow name:

- Unit operations:

- Output:

### Run Record (Code Cell)

- run_id

- operator

- date/time

- notes_pre_run

- notes_during_run

- notes_deviations

- notes_post_run

### Required Sections

- Parameters (single source of truth)

- Steps (unit operations list)

- Unit operation sections (with validation & deviations)

- Validation checks and results

- Export / artifacts

- Change log